Converting Spotwelders from 220v to 440v

Step 1: Remove cover plates on the control side of the machine (Picture 1)

Step 2: On the back side of the control coverplate, you’ll see a black square transformer (visible in the top right corner of Picture 2)

Step 3: Remove the black wire from the “230” input and install it into the “440” input, keep the “0” wire where it is. (Visible in Picture 3)

Step 4: Reinstall the control cover plate.

Step 5: Below the control cover plate, you’ll see the main power inputs. From top to bottom they are stamped as Ground symbol, 5, 4, 3, 2, 1. (See Picture 4)

Step 6: Move the black wire in the “2” input into the “3” input. (See picture 5).

Step 7: Re-install the main cover plate.